# PMS / Service / Maintenance Intervals — Authoritative Mailbox Sources Only

Created: 2026-07-27

Author filter (hard rule): only intervals traceable to mail and/or PDF attachments from:

| Person | Role / entity | Email (from Outlook) |
| --- | --- | --- |
| Duncan Young | Parts Director, Isuzu Truck (UK) Ltd (ITUK) | duncan.young@isuzutruck.co.uk |
| Takayoshi Suzuki | ISZJ / IML International Parts | Takayoshi_Suzuki@isuzu.com |
| René Paul | Aftersales Service Specialist, Isuzu Motors Germany GmbH (IMG) | rene.paul@isuzu.de |

Search method: Outlook COM across Jerome’s mailbox stores (jerome.van.der.pluym@isuzu-e.com, Sales, Parts), Restrict filters on sender name/email, subject search for Interval maintenance / maintenance schedule / service interval / PMS interval, PDF attachment save + text/visual extraction.

Deliverable path:

C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\PMS_Intervals_Authoritative_Sources.md

Working pull folder (attachments + OCR/render helpers):

C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\

# Executive result

| Sender | Interval numbers found? | Primary evidence |
| --- | --- | --- |
| René Paul (IMG) | YES — GIDSS workshop-manual maintenance schedules (N & F series, MY14 & MY24) | Email 2026-07-16 + 4 PDFs |
| Duncan Young (ITUK) | NO numeric service intervals | 12 inbox mails; 1 PDF (strategy/accessories, no PMS km grid) |
| Takayoshi Suzuki (ISZJ/IML) | NO numeric service intervals | 19 inbox mails; PMS = parts promotion / APL / BVP filters, not service schedule |

Interval entries in this document: ~55+ discrete replace/inspect intervals with explicit km/mi/months (René / GIDSS only).

Explicit gaps: listed in §4 (ITUK visits/yr, IML schedule PDF from Suzuki, D-Max/RT schedules only from Georg Fischer on same thread — excluded).

# 1. René Paul (IMG) — authoritative

## 1.1 Source email

| Field | Value |
| --- | --- |
| Sender | René Paul (Rene.Paul@isuzu.de / rene.paul@isuzu.de) |
| Date | 2026-07-16 07:24 |
| Subject | Re: Interval maintenance |
| Source type | Email + PDF attachments |
| Folder | \\jerome.van.der.pluym@isuzu-e.com\Inbox |
| Outlook EntryID | …0008C5EDB49F0000 (full ID in pms_intervals_mail_pull\pdf_hits.json) |
| Context | Jerome asked (2026-07-15) for N & F series (P700 + previous model) schedules; René replied with GIDSS workshop-manual PDF prints. |

Attachments (saved under):

C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\attachments\

| Attachment filename | Saved as |
| --- | --- |
| GIDSS_maint_N_MY24.pdf | 20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY24.pdf |
| GIDSS_maint_N_MY14.pdf | 20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY14.pdf |
| GIDSS_maint_F_MY24.pdf | 20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY24.pdf |
| GIDSS_maint_F_MY14.pdf | 20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY14.pdf |

Legend (all GIDSS schedules): I = Inspect; A = Adjust; R = Replace; T = Tighten; L = Lubricate.

Intervals are typically odometer or months, whichever comes first.

## 1.2 Market / model: N-series MY24 (P700 era) — `GIDSS_maint_N_MY24.pdf`

PDF: C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\attachments\20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY24.pdf

Source type: PDF (from René)

Sender / date / subject: René Paul · 2026-07-16 · Re: Interval maintenance

Attachment: GIDSS_maint_N_MY24.pdf

### A) NLR88 / NMR88 / NPR88 — **4JZ1** (pages 1–3)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil | When “engine oil and filter indicator light” displayed or every 12 months | alert / months | R | Note *2: light illuminates by max 40,000 km (24,000 mi) |
| Engine oil filter | Same as engine oil | alert / months | R | Same *2 cap |
| Fuel filter element | 40,000 (also 80/120/160) | km | R | or every 12 months; Severe X |
| Fuel filter element | 24,000 (49/75/99) | miles | R | same row |
| Pre-fuel filter | 40,000 cycle | km | R | or every 12 months; Severe X |
| Air cleaner element | I at 20/60/100/140; R at 40/80/120/160 | ×1,000 km | I/R | or every 24 months R; Severe X |
| Drive belt | every 20,000 | km | I | or every 12 months |
| Engine coolant | every 12 mo I; every 24 mo R | months | I/R | when Isuzu recommended coolant used |
| Brake fluid | 40,000 | km (24,000 mi) | R | or every 24 months |
| Brake lining/pad/fluid level | every 20,000 | km (12,000 mi) | I | or every 12 months; Severe X on several items |
| Transmission oil (MYY / MYR) | R every 40,000 | km (24,000 mi) | R | I on alternate 20k steps; or every 24 months; Severe X |
| [M/T] Clutch fluid | R every 40,000 | km | R | I alternate 20k; replace every 24 months |
| [DCT] DCT clutch oil | R every 40,000 | km | R | or every 24 months; Severe X |
| Differential gear oil | R every 40,000 | km | R | or every 24 months; Severe X |
| PM sensor | 240,000 | km (149,000 mi) | R | or every 180 months |
| NOx sensor | 280,000 | km (174,000 mi) | R | or every 180 months |
| O2 sensor | 240,000 | km (149,000 mi) | R | or every 180 months |

Grid base for this schedule: columns at 20 / 40 / 60 / 80 / 100 / 120 / 140 / 160 ×1,000 km (12 / 24 / 37 / 49 / 62 / 75 / 87 / 99 ×1,000 miles).

### B) NPR75 — **4HK1** / N75 (pages 3–5) — *N75 flagship for ITUK PMS math*

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil (except Israel) | 40,000 | km | R | at 40/80/120/160; or every 12 months; Severe X |
| Engine oil (except Israel) | 24,000 | miles | R | same |
| Engine oil (Israel) | 22,500 | km (14,000 mi) | R | or every 12 months; Severe X |
| Engine oil filter | same as oil (ex-Israel / Israel) | km / mi | R | Severe X |
| Fuel filter element | 40,000 cycle | km | R | or every 12 months; also when fuel-filter indicator light on |
| Pre-fuel filter | when fuel-filter indicator light | alert | R |  |
| Air cleaner element | when air-cleaner inspection indicator light | alert | I (else) | indicator-driven |
| AdBlue filter | 200,000 | km (124,000 mi) | R | Severe X |
| Brake fluid | 40,000 | km (24,000 mi) | R | or every 24 months |
| Brake pad / fluid level inspect | every 20,000 | km | I | Severe X on pad/fluid |
| Transmission oil (MZZ) | R every 40,000 | km | R | or every 24 months; Severe X |
| [M/T] Clutch fluid | R every 40,000 | km | R | or every 24 months |
| [SA] Smoother clutch oil | R every 40,000 | km | R | or every 24 months; Severe X |
| Differential gear oil | R every 40,000 | km | R | or every 24 months; Severe X |
| Power steering fluid | 40,000 | km | R | or every 24 months |
| Power steering hose | 80,000 | km (49,000 mi) | R | or every 48 months |
| PM sensor | 240,000 | km | R | or every 180 months |
| NOx sensor | 280,000 | km | R | or every 180 months |

### C) NLR87 — **RZ4E** (pages 6–8)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil | 20,000 | km (12,000 mi) | R | every 20k step; or every 12 months; Severe X |
| Engine oil filter | 20,000 | km | R | same; Severe X |
| Oil/filter alert (*2 on later note page) | if alert shown at &lt; 20,000 km (12,000 mi) | km | R | follow indicated interval |
| Fuel filter element | 40,000 cycle | km | R | or every 12 months |
| Pre-fuel filter | 40,000 cycle | km | R | or every 12 months |
| Air cleaner element | R at 60 / 120; I other 20k steps | ×1,000 km | I/R | or every 24 months R; Severe X |
| Drive belt | every 20,000 | km | I | or every 12 months |
| PM sensor | 240,000 | km | R | or every 180 months |
| NOx sensor | 300,000 | km (186,000 mi) | R | or every 180 months |
| O2 sensor | 240,000 | km | R | or every 120 months (per table) |
| Power steering fluid | 40,000 | km | R | or every 24 months |

## 1.3 Market / model: N-series MY14 (previous) — `GIDSS_maint_N_MY14.pdf`

PDF: ...\attachments\20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY14.pdf

Sender / date / subject / attachment: René Paul · 2026-07-16 · Re: Interval maintenance · GIDSS_maint_N_MY14.pdf

### A) Normal schedule — NLR85 / NLS85 / NMR85 / NNR85 / NPR85 (models No. 1–2)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil (Israel) | 10,000 | km | R | or every 12 months |
| Engine oil (no PTO, except Israel) | 15,000 | km | R | or every 12 months |
| Engine oil (with PTO, except Israel) | 10,000 | km | R | or every 12 months |
| Engine oil filter | same splits as oil | km | R | Israel / PTO / no-PTO |
| Fuel filter element | 40,000 | km | R | or every 12 months |
| Pre-fuel filter | 40,000 | km | R | or every 24 months |
| Air cleaner element | R every 40,000; I every 10,000 | km | I/R | or every 24 months |
| Clutch fluid [M/T] | R every 45,000 | km | R | I every ~20k; or every 27 months |
| Transmission oil | R every 45,000 | km | R | or every 24 months |
| Smoother clutch oil | R every 45,000 | km | R | or every 24 months |
| Differential gear oil | R every 45,000 | km | R | or every 24 months |

### B) NPR75 model No. 1–2 (chassis continuation pages)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Brake fluid | 45,000 / 90,000 | km | R | or every 27 months |
| Power steering fluid | 40,000 / 80,000 | km | R | or every 24 months |
| Power steering hose | 80,000 | km | R | or every 48 months |
| Wheel hub bearing grease | 40,000 / 80,000 | km | R | or every 24 months |
| AdBlue filter (if equipped) | 200,000 | km (124,000 mi) | R |  |
| Kingpin (rigid front) | every 10,000 | km | L | or every 12 months |
| Brake system inspect | ~ every 5,000–10,000 | km | I | table dense; see PDF p2–p3 |

### C) Severe-condition schedule (PDF text extract, pages 4–5)

| Category / part | Value | Units | Action | Engine / note |
| --- | --- | --- | --- | --- |
| Engine oil | 10,000 | km (6,000 mi) | R | 4JJ1 without PTO (except Israel) — conditions A/B |
| Engine oil | 7,500 | km (4,500 mi) | R | 4JJ1 with PTO (except Israel); also 4JJ1 Israel |
| Engine oil | 10,000 | km (6,000 mi) | R | 4HK1 |
| Engine oil filter | same as oil rows above | km | R | 4JJ1 / 4HK1 splits |
| Air cleaner element | I every 5,000; R every 20,000 | km | I/R | dusty (C) marked |
| Transmission oil | 20,000 | km (12,000 mi) | R |  |
| Smoother clutch oil | 20,000 | km | R |  |
| Differential gear oil | 20,000 | km | R |  |
| Brake shoe/drum wear | I every 10,000 | km | I |  |
| Disc pad/disc wear | I every 5,000 | km | I |  |

## 1.4 Market / model: F-series MY24 — `GIDSS_maint_F_MY24.pdf`

PDF: ...\attachments\20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY24.pdf

Sender / date / subject / attachment: René Paul · 2026-07-16 · Re: Interval maintenance · GIDSS_maint_F_MY24.pdf

### A) Italy, Israel, Portugal, United Kingdom, Ireland — **4HK1** (section header on p1)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil | when “engine oil and filter indicator light” on MID | alert | R | Severe X |
| Engine oil filter (main + partial) | same indicator | alert | R | Severe X |
| Fuel filter (engine-side + chassis-side) | when “fuel filter indicator light” on MID | alert | R |  |
| Air cleaner element | when “air cleaner inspection indicator light” on MID | alert | R (else I) |  |
| Oil separator element | every 12 | months | R |  |
| Fuel hose | every 48 | months | R |  |
| Engine coolant | 12 mo I; 24 mo R | months | I/R | Isuzu recommended coolant |
| AdBlue filter (4HK1-TCH) | 80,000 | km (50,000 mi) | R | Severe X |
| DPD pressure check / filter cleaning | 100,000 | km (62,000 mi) | I | or every 12 months |
| PM sensor | 240,000 | km (149,000 mi) | R |  |
| NOx sensor | 280,000 | km (174,000 mi) | R |  |
| Brake fluid | R every 40,000; I alternate 20k | km | I/R | or every 12 months (table) |
| Brake lining wear | every 20,000 | km | I | or every 12 months; Severe X |
| Brake hose | R at 80,000 / 160,000 | km | R | or every 24 months |
| ABS modulator rubber parts | every 24 | months | R |  |
| Brake chamber piggy-back | every 36 | months | R |  |
| Brake expander overhaul | every 36 | months | OH |  |
| Brake valve / EPM | every 10 | years | R |  |

### B) Australia / New Zealand — **4HK1** & **DB6A** (later sections)

Header text in PDF (extractable): schedules split by fuel consumption ≥ / &lt; 18.2 L/100 km (4HK1), plus separate DB6A block.

Transmission oil (ES-9306A) extractable text includes:

| Category / part | Value | Units | Notes |
| --- | --- | --- | --- |
| Oil level check (linehaul) | up to 20,000 | km (12,000 mi) |  |
| Manual TX oil change — Extended Eaton PS-386 | 400,000 | km (248,000 mi) | GCW < 50 t |
| Manual TX oil change — Standard Eaton PS-321 | 120,000 | km (74,000 mi) |  |
| MTF Gear Oil API GL-4 | 100,000 | km (62,000 mi) |  |
| First TX oil change (*3 note) | initial 5,000 | km (3,000 mi) | Italy/UK block note also cites this |

Differential / Meritor axle (text extract p23): petroleum oil change linehaul 160,000 km / city 80,000 km / vocational 40,000 km (or annually); synthetic much longer (up to 800,000 km linehaul).

Full engine-oil km grids for AU/NZ pages are largely image-only; treat indicator-driven EU/UK/IL/PT/IE block above as the confirmed F-MY24 4HK1 European set from this PDF.

## 1.5 Market / model: F-series MY14 — `GIDSS_maint_F_MY14.pdf`

PDF: ...\attachments\20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY14.pdf

Sender / date / subject / attachment: René Paul · 2026-07-16 · Re: Interval maintenance · GIDSS_maint_F_MY14.pdf

### A) Normal schedule (page 1 table — dense 5.5k / 11k-style columns)

| Category / part | Value | Units | Action | Notes |
| --- | --- | --- | --- | --- |
| Engine oil | R on ~11,000 km pattern; I on mid steps | km | I/R | time: I every 3 months; R every 6 months |
| Engine oil filters (main/partial/combined/separator) | R every 22,000 | km | R | or every 6 months |
| Fuel filter | R at 44,000 / 88,000 | km | R | or every 9 months |
| Air cleaner element | R at 44,000 and 104,500 | km | R | also time-based I/R rules on table |
| Clutch fluid [M/T] | R at 55,000 / 115,500 | km | R | I frequent; R every 15 months |
| Engine coolant | every 24 | months | R |  |

### B) Severe-condition (text extract pages 5–6) — mentions **4HK1** oil separator

| Category / part | Value | Units | Action | Conditions |
| --- | --- | --- | --- | --- |
| Air cleaner element | 24,000 | km (15,000 mi) or after 6th cleaning | R | dusty (C) |
| Engine oil | 10,000 | km (6,000 mi) | Change | A/B/C marked |
| Engine oil filter | 10,000 | km | Change | A/B/C |
| Engine oil separator (4HK1) | 10,000 | km | Change | A/B/C |
| Transmission oil | 24,000 | km (15,000 mi) | R | A/B |
| Clutch fluid [M/T] | 24,000 | km | Change | A |
| Smoother clutch oil | 24,000 | km | Change | A/B |
| Rear axle differential oil (Isuzu axle) | 24,000 | km | Change | A/B |
| Brake lining wear | I every 2,500 | km (1,500 mi) | I | A/B/C/D |
| Brake drum wear | I every 20,000 | km | I | A/B/C/D |

# 2. Duncan Young (ITUK) — searched; **no interval numbers**

## 2.1 Mailbox hits (Inbox Restrict on “Duncan”)

12 messages from Duncan Young / duncan.young@isuzutruck.co.uk (Jun–Jul 2026 sample in live Inbox). Subjects include P700 order, BPFY revision, Parts Month End, Meeting today, engine VIN queries, etc.

Keyword scan of full bodies: no oil change / xx,000 km / air-fuel filter service interval statements.

## 2.2 PDF from Duncan

| Field | Value |
| --- | --- |
| Date | 2026-07-23 14:41 |
| Subject | RE: Meeting today |
| Attachment | Parts Business & Service product.pdf |
| Saved as | C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\attachments\20260723_Duncan_Young_Parts_Business_Service_product.pdf |
| Content | IML/ISZE parts strategy, Vision 2030, R&M programme framing, D-Max accessories — no PMS km/mi service grid |

## 2.3 What Duncan mail *does* say (non-interval)

Mentions PMS Parts as a discussion point (RE: P700 order, 2026-07-20).

No oil/air/fuel/brake/clutch interval values.

→ Duncan section: zero authoritative interval entries.

# 3. Takayoshi Suzuki (ISZJ / IML) — searched; **no interval numbers**

## 3.1 Mailbox hits

19 messages from Takayoshi Suzuki ISZJ / Takayoshi_Suzuki@isuzu.com in Inbox (Restrict). Topics: PMS parts promotion, BVP air filters, APL zip, SPA, meetings, Russian steel priority lists, SPI sheets.

## 3.2 Closest PMS-adjacent content (not service intervals)

| Date | Subject | What it contains | Interval? |
| --- | --- | --- | --- |
| 2026-07-27 / 07-24 | Demand forecast PN 5876102500 ELEMENT AIR CLEANER | BVP filter on PMS Parts Promotion list; ITUK awareness | No km interval |
| 2026-07-23 | Recommended parts list ITUK | APL zip 20260714_1037.zip | Parts list, not schedule |
| 2026-07-24 | RE: Meeting | PMS promotion schedule for Aug 18 meeting | Meeting agenda, not service km |

→ Suzuki section: zero authoritative interval entries.

IML “official” service schedule PDF was not found as an attachment from Suzuki in this mailbox pull.

# 4. Explicit gaps (do **not** treat as authoritative under the three-sender rule)

| Gap | Why excluded / missing |  |  |
| --- | --- | --- | --- |
| D-Max / RT56 / RT88-93 / RG / RG21 / BEV RG20 Service Maintenance Schedules | Present in mailbox on same thread (Interval maintenance) but sent by Georg Fischer (Georg.Fischer@isuzu.de, 2026-07-15), not René/Duncan/Suzuki. PDFs saved for reference under attachments\20260715_Georg_Fischer_* but not entered as interval rows above. |  |  |
| ITUK “visits per year” / annual service visit count | Not stated in Duncan mail/PDF in this search. |  |  |
| IML factory schedule PDF from Suzuki / ISZJ | Not found as Suzuki attachment. René’s GIDSS prints are workshop-manual extracts (IML copyright on footers), but channelled via IMG René, not Suzuki mail. |  |  |
| N35-specific schedule | Not labelled “N35” in René PDFs; closest is N-series MY14/MY24 model codes (NLR/NMR/NPR…). |  |  |
| Online Archive deep history | Outlook Restrict returned 0 Duncan/Suzuki/René hits under `Archive\…\customers\ITUK | IMG | IML` in this pass (likely incomplete local sync). Live Inbox is where the GIDSS PDFs were found. Older interval mails may exist only in online archive / another mailbox. |
| Jerome’s own interval models (xlsx/docx) | Present in Sent (PMS, IML PMS) — excluded by design (not from the three senders). |  |  |

# 5. Quick reference — PMS-critical replace intervals (René / GIDSS only)

Useful for N75 / 4HK1 / 4JZ1 potential calculations. All from René 2026-07-16 PDFs.

| Vehicle / engine | Oil change | Oil filter | Air cleaner | Fuel filter | Brake fluid | Clutch fluid / oil |
| --- | --- | --- | --- | --- | --- | --- |
| NPR75 MY24 4HK1 (N75) | 40,000 km / 12 mo (Israel 22,500 km) | same | indicator | 40,000 km / 12 mo (+ indicator) | 40,000 km / 24 mo | 40,000 km / 24 mo |
| NLR/NMR/NPR88 MY24 4JZ1 | indicator (max 40,000 km) / 12 mo | same | R 40,000 km | 40,000 km / 12 mo | 40,000 km / 24 mo | 40,000 km / 24 mo |
| NLR87 MY24 RZ4E | 20,000 km / 12 mo (alert may be earlier) | same | R 60,000 km | 40,000 km / 12 mo | *(see PDF chassis pages)* | *(see PDF)* |
| N85 MY14 family | 10k / 15k / 10k km (Israel / no-PTO / PTO) | same | R 40,000 km | 40,000 km | *(chassis pages)* | 45,000 km clutch |
| N MY14 severe 4HK1 | 10,000 km | 10,000 km | R 20,000 km | *(see PDF)* | inspect denser | TX 20,000 km |
| F MY24 4HK1 (IT/IL/PT/UK/IE) | MID oil indicator | same | MID air indicator | MID fuel indicator | R 40,000 km | *(F brake-heavy pages)* |
| F MY14 severe | 10,000 km | 10,000 km | 24,000 km / 6th clean | *(normal table 44k)* | lining 2,500 km I | clutch 24,000 km |

# 6. Provenance checklist (per user rules)

For every interval row in §§1.2–1.5:

☑ Value + unit recorded

☑ Part / category recorded

☑ Vehicle / model / engine recorded when stated on the PDF page

☑ Source type = PDF (René)

☑ Sender = René Paul

☑ Date = 2026-07-16

☑ Subject = Re: Interval maintenance

☑ Attachment filename = one of the four GIDSS_maint_*.pdf names

Duncan / Suzuki: searched; no interval rows added.

# 7. Files created during this pull

| Path | Purpose |
| --- | --- |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\PMS_Intervals_Authoritative_Sources.md | This reference |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\attachments\ | PDFs from René (+ Duncan strategy PDF; Georg out-of-scope copies) |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\pdf_pages\ | Rendered PNG pages of René GIDSS PDFs |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\sender_hits.json | Outlook sender Restrict hits |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\fullbody_Re_Interval_maintenance.txt | Full René thread body |

End of authoritative extract. Do not merge Georg Fischer or local PC spreadsheet intervals into this list without a new mail/PDF from Duncan, Suzuki, or René.

# Appendix A — Source email thread (René Paul, 2026-07-16)

Verbatim body from pms_intervals_mail_pull\Rene_20260716_0724_Re_Interval_maintenance.txt. Included for provenance only. Georg Fischer content on this thread is out of scope under the three-sender rule (see §4 gaps).

﻿HI Jerome,

please find below the pdf prints from the GIDSS workshop manual.

(See attached file: GIDSS_maint_F_MY14.pdf)(See attached file: GIDSS_maint_F_MY24.pdf)(See attached file: GIDSS_maint_N_MY14.pdf)(See attached file: GIDSS_maint_N_MY24.pdf)

Best Regards,

Mit freundlichen Grüßen,

Yoroshiku Onegaishimasu,

René Paul

Aftersales Service Specialist

Aftersales Department

ISUZU MOTORS Germany GmbH

Im Weiherfeld 2

65462 Ginsheim-Gustavsburg

GERMANY

Tel.:      +49 - (0) 6134 - 558-456 / Fax: - 410

Mobile: +49 – (0) 151 – 171  55 180

E Mail : rene.paul@isuzu.de

Sitz der Gesellschaft: Ginsheim-Gustavsburg

Amtsgericht Darmstadt · HRB 53969 · Ust.-ID-Nr.: DE 192006605

Geschäftsführer: Norihiro Yoshimoto, Ottmar Degrell, Hidezo Umiyama

"Jerome Van Der Pluym" ---15.07.2026 18:43:39---Dear Rene, Can you provide the same documents for N & F series (P700 + previous model)

From: "Jerome Van Der Pluym" <jerome.van.der.pluym@isuzu-e.com>

To: "'Rene.Paul@isuzu.de'" <rene.paul@isuzu.de>

Cc: "'Antonio.Pfeifer@isuzu.de'" <antonio.pfeifer@isuzu.de>, "Bart Symons" <bart.symons@isuzu-e.com>, "Koen Matheussen" <koen.matheussen@isuzu-e.com>, "georg.fischer" <georg.fischer@isuzu.de>

Date: 15.07.2026 18:43

Subject: Re: Interval maintenance

________________________________

Dear Rene,

Can you provide the same documents for N & F series (P700 + previous model)

Many thanks and kind regards,

Jérôme Van der Pluym

Assistant Manager // Pricing analyst

Sales – Vehicle Parts | Isuzu Motors Europe NV

+32 475 851 089

jerome.van.der.pluym@isuzu-e.com <mailto:jerome.van.der.pluym@isuzu-e.com>

Bist 12, 2630 Aartselaar, Belgium

________________________________

From: Georg.Fischer@isuzu.de <Georg.Fischer@isuzu.de>

Sent: Wednesday, July 15, 2026 8:16 AM

To: Jerome Van Der Pluym <jerome.van.der.pluym@isuzu-e.com>

Cc: 'Antonio.Pfeifer@isuzu.de' <antonio.pfeifer@isuzu.de>; Bart Symons <bart.symons@isuzu-e.com>; Koen Matheussen <koen.matheussen@isuzu-e.com>

Subject: Re: Interval maintenance

Good morning Jerome,

thanks yes and hope you too.

Not 100% sure but I remember that I've already send Service Maintenance Data once in the past, except BEV, but below

all again because have them stored at this time

(See attached file: RT56_Service Maintenance Schedule.pdf), (See attached file: RT88_93_Service Maintenance Schedule.pdf),

(See attached file: RG_Service Maintenance Schedule.pdf), (See attached file: RG21_Service Maintenance Schedule.pdf)

(See attached file: BEV_RG20_Service Maintenance Schedule.pdf)

Hope that helps and wish you a nice day.

Greetings

Georg

Best Regards,

Mit freundlichen Grüßen,

Yoroshiku Onegaishimasu,

Georg Fischer

Aftersales Service Specialist

Aftersales Department

ISUZU MOTORS Germany GmbH

Im Weiherfeld 2

65462 Ginsheim-Gustavsburg

GERMANY

Tel.:      +49 - (0) 6134 - 558-401 / Fax: - 410

Mobile: +49 – (0) 151 – 171  55 106

E Mail : georg.fischer@isuzu.de

Internet: http://www.isuzu.de <https://cas5-0-urlprotect.trendmicro.com:443/wis/clicktime/v1/query?url=http%3a%2f%2fwww.isuzu.de&umid=e5858d9a-8014-11f1-90f4-70a8a5889283&rct=1784096264&auth=c409b3fe49adaa4cc1568465e413bd2c45612ee0-754dae2f2a58dfa1bb89ab01140a877ca2f8ecb8>

Sitz der Gesellschaft: Ginsheim-Gustavsburg

Amtsgericht Darmstadt · HRB 53969 · Ust.-ID-Nr.: DE 192006605

Geschäftsführer: Norihiro Yoshimoto, Ottmar Degrell, Hidezo Umiyama

"Jerome Van Der Pluym" ---14.07.2026 16:58:24---Hello Georg & Antonio, I hope you are fine.

From: "Jerome Van Der Pluym" <jerome.van.der.pluym@isuzu-e.com>

To: "georg.fischer" <georg.fischer@isuzu.de>, "'Antonio.Pfeifer@isuzu.de'" <antonio.pfeifer@isuzu.de>

Cc: "Koen Matheussen" <koen.matheussen@isuzu-e.com>, "Bart Symons" <bart.symons@isuzu-e.com>

Date: 14.07.2026 16:58

Subject: Interval maintenance

________________________________

Hello Georg & Antonio,

I hope you are fine.

Could you share the interval schedules for Dmax?

If you have the ones for older models RT they are also welcome.

We are trying to start the calculate the market potential for maintenance parts.

Many thanks and kind regards,

Jérôme Van der Pluym

Assistant Manager // Pricing analyst

Sales – Vehicle Parts | Isuzu Motors Europe NV

+32 475 851 089

jerome.van.der.pluym@isuzu-e.com <mailto:jerome.van.der.pluym@isuzu-e.com>

Bist 12, 2630 Aartselaar, Belgium

# Appendix B — Attachment inventory (mail pull)

Files present under pms_intervals_mail_pull\attachments\ at export time. Georg Fischer PDFs listed for reference only — not used as interval sources.

| Saved filename | Scope |
| --- | --- |
| 20260715_Georg_Fischer_isuzu_de_BEV_RG20_Service_Maintenance_Schedule.pdf | Out of scope (not René/Duncan/Suzuki) |
| 20260715_Georg_Fischer_isuzu_de_RG21_Service_Maintenance_Schedule.pdf | Out of scope (not René/Duncan/Suzuki) |
| 20260715_Georg_Fischer_isuzu_de_RG_Service_Maintenance_Schedule.pdf | Out of scope (not René/Duncan/Suzuki) |
| 20260715_Georg_Fischer_isuzu_de_RT56_Service_Maintenance_Schedule.pdf | Out of scope (not René/Duncan/Suzuki) |
| 20260715_Georg_Fischer_isuzu_de_RT88_93_Service_Maintenance_Schedule.pdf | Out of scope (not René/Duncan/Suzuki) |
| 20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY14.pdf | Authoritative (René / GIDSS) |
| 20260716_Rene_Paul_isuzu_de_GIDSS_maint_F_MY24.pdf | Authoritative (René / GIDSS) |
| 20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY14.pdf | Authoritative (René / GIDSS) |
| 20260716_Rene_Paul_isuzu_de_GIDSS_maint_N_MY24.pdf | Authoritative (René / GIDSS) |
| 20260723_Duncan_Young_Parts_Business_Service_product.pdf | Searched — no PMS km grid |

# Appendix C — Mail-pull working files

| Path / file | Purpose |
| --- | --- |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\sender_hits.json | Outlook sender Restrict hits |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\fullbody_Re_Interval_maintenance.txt | Full René thread body (duplicate of Appendix A source) |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\pdf_text | PyMuPDF text extracts of René (+ Duncan) PDFs |
| C:\Users\jerom\Desktop\claude\ISZE\01_data\90_INBOX\pms_intervals_mail_pull\pdf_pages | Rendered PNG pages of René GIDSS PDFs |

Export note: This Word/PDF is a faithful export of PMS_Intervals_Authoritative_Sources.md plus provenance appendices. No new interval numbers were added.